Conceptual & Logical Network Design
Secure Multi-Site Network for SCG Hobby
Project #: IT499-NET-001
Prepared by: Christian Mclean
Date: March 8, 20261.

This conceptual and logical design creates a secure, centralized, and scalable network connecting four hobby shop locations in western Pennsylvania. Core Architecture:

- Hub-and-Spoke VPN Topology using Ubiquiti Dream Machine Pro SE gateways

- One central “hub” site (or cloud-managed controller) with four spoke sites

- All inter-site traffic travels through encrypted site-to-site VPN tunnels

- Centralized management via UniFi Controller (Cloud Key Gen2 Plus)

- Designed for future expansion (1–2 additional sites) with zero major rework

2. Logical Network Diagram

3. VLAN Segmentation

| VLAN ID | Name | Purpose | Access Restrictions |
| --- | --- | --- | --- |
| 10 | Staff & POS | POS terminals, inventory system | Restricted – only authorized devices |
| 20 | Guest Wi-Fi | Customer/visitor Wi-Fi | Fully isolated – Internet only |
| 30 | Security & IoT | IP cameras, alarms, sensors | No access to POS or staff VLAN |
| 40 | Management | UniFi Controller, switches, gateways | Admin access only (MFA required) |

4. Key Logical Features

- Site-to-Site VPN: IPsec/WireGuard tunnels between all sites (encrypted, always-on)

- Inter-VLAN Routing: Blocked by default – only explicit firewall rules allow necessary traffic like POS server to inventory

- Remote Access: Managers connect via VPN + MFA

- IP Addressing: Private 10.0.0.0/8 scheme with /24 subnets per site

- DNS & DHCP: Handled centrally by UniFi Dream Machine gateways

- Monitoring: All traffic logged and visible in UniFi dashboard

5. Scalability & Future-Proofing

- Easy addition of new sites by simply adding a new spoke gateway and VPN tunnel

- Cloud Key supports up to 50+ devices room for growth

- Design supports future cloud backup integration already added via change request