Physical and Logical Security Plan
Secure Multi-Site Network for SCG Hobby
Project #: IT499-NET-001
Prepared by: Christian Mclean
Date: March 8, 2026

- This comprehensive security plan defines the logical and physical controls required to protect SCG Hobby’s new multi-site network. The plan addresses the risks identified in the Project Charter (data breaches, inventory theft, unauthorized access, and operational silos) by combining the UniFi ecosystem, site-to-site VPN, IP cameras, alarms, and cloud backups.

The goal is to create a secure, scalable environment that:

- Enables real-time POS and inventory synchronization across four locations

- Protects customer data and physical assets

- Meets basic compliance expectations for a small retail business

- Supports future expansion with minimal rework

All controls are designed to be cost-effective, centrally managed, and easy for non-technical staff to follow.

2. Logical Security Plan

2.1 Network Segmentation (VLANs)

- VLAN 10 – Staff & POS Systems (restricted access)

- VLAN 20 – Guest Wi-Fi (fully isolated, no access to internal resources)

- VLAN 30 – Security & IoT (cameras, alarms, sensors)

- VLAN 40 – Management (UniFi Controller, switches, gateways – admin access only)

- Inter-VLAN routing is blocked except for explicitly allowed traffic (e.g., POS to inventory server)

2.2 Access Controls & Authentication

- Role-Based Access Control (RBAC) enforced via UniFi

- Multi-Factor Authentication (MFA) required for all accounts

- Least-privilege principle applied to every user and device

2.3 Remote Access & VPN

- Site-to-site VPN using IPsec/WireGuard on Dream Machine Pro SE gateways

- All remote manager access requires VPN + MFA

- VPN traffic is encrypted end-to-end and logged for 90 days

2.4 Backup & Disaster Recovery

- Daily automated encrypted backups of POS, inventory, and camera footage

- Backups stored locally on Cloud Key Gen2 Plus and replicated to secure cloud provider

- 30-day retention with point-in-time restore capability

- Monthly full restore test documented and reviewed

2.5 Monitoring & Logging

- UniFi dashboard provides centralized logging of all network events

- Real-time alerts for failed logins, unusual traffic, or camera offline events

- Weekly log review by project sponsor or designated manager

3. Physical Security Plan

3.1 Video Surveillance

- 6 × UniFi G4 Pro 4K PoE cameras per site

- Strategic placement: 2 exterior (parking/entrances), 2 interior (cash registers), 1 storage area, 1 back door

- Features: night vision, motion detection, person detection, 14-day local retention + cloud archive

- Tamper alerts sent to manager phones

3.2 Intrusion Detection & Alarms

- One alarm panel + 8 sensors per site (door contacts, motion detectors, glass-break sensors)

- Automatic arming at close of business via schedule

- Silent alarm to manager + local siren option

- Integration with UniFi Protect for unified alerts

3.3 Physical Access Controls

- All network closets and equipment racks use keyed locks (keys held by managers only)

- PoE switches and gateways mounted in locked 6U wall cabinets

- Server room access logged via camera footage

3.4 Environmental Controls

- UPS units on all critical devices (gateways, switches, NVR) for 30+ minutes runtime

- Surge protection on every PoE run

4. Integrated Controls & Threat Mitigation

| Threat | Likelihood | Impact | Risk Score | Color Code | Mitigation Strategy | Owner |
| --- | --- | --- | --- | --- | --- | --- |
| Data breach via POS system | Medium | High | High | Red | VLAN isolation + MFA + encrypted cloud backups | Store Manager |
| Physical theft of equipment/data | High | High | High | Red | IP cameras + alarms + locked network closets | Store Manager |
| Unauthorized remote access | Low | High | Medium | Yellow | Site-to-site VPN with MFA + Zero Trust enforcement | Store Manager |
| Camera or alarm tampering | Medium | Medium | Medium | Yellow | Motion detection alerts + tamper sensors | Store Manager |
| Natural disaster / power outage | Low | Medium | Low | Green | UPS on critical devices + daily cloud backups | Store Manager |
| Malware / ransomware infection | Medium | High | High | Red | UniFi security features + endpoint protection | Store Manager |

Color Legend

- Red = High Risk – Immediate action required

- Yellow = Medium Risk – Monitor closely

- Green = Low Risk – Standard controls sufficient

5. Monitoring, Maintenance & Compliance

- Weekly security dashboard review (UniFi Protect + logs)

- Monthly backup restore test + alarm system check

- Quarterly full security audit

- Annual policy review and staff retraining

- Incident response process: report within 1 hour → notify sponsor → document in shared drive

6. Approval

Sponsor:
Christian Mclean _______________________________ Date: _______________